- REPORT ON

INTELLIGENT HUMIDISTAT

BY

| Meet Kanani | 2017A7PS0128P |
| --- | --- |
| Aditya Ramaswamy | 2017A7PS0130P |
| Mukkamala Venkat Sai Ram | 2017A7PS0133P |
| S Hariharan | 2017A7PS0134P |

Prepared in partial fulfilment of the requirements of the course “Microprocessors and Interfacing” - Course Number: CS F241

AT

BIRLA INSTITUTE OF TECHNOLOGY & SCIENCE, PILANI

BIRLA INSTITUTE OF TECHNOLOGY & SCIENCE, PILANI

PILANI – 333031 (RAJASTHAN)

Submission Date and Place: 25th April 2019, Pilani

| Title of the Project: Intelligent Humidistat |  |
| --- | --- |
| Project Number: 23 Group Number: 91 |  |

Name of Experts: Dr. Nitin Chaturvedi

Designation: Instructor-in-Charge (ICs)

Key Words: Microprocessors, Interfacing, Peripherals like ADC, RAM, Octal Buffers, Latches, LCD Display, Decoders, etc.

Project Areas: Microprocessors and Interfacing, Assembly Language Programming

Abstract: This project aims to make an Intelligent Humidistat device. As per the problem statement, we design and emulate the hardware and block diagram of this device in a software called “Proteus”. Various external components are to be used like sensors, potentiometers, RAMs, ROMs, decoders, etc. Using this hardware design and block diagram, we prepare a flowchart on how the system will work when programmed. Following that chart, we thereby write a program in Assembly Language for the device using MASM 611.

|  |  | TABLE OF CONTENTS |  |
| --- | --- | --- | --- |
| 1. | Problem Statement | 1 |  |
| 2. | Assumptions | 1 |  |
| 3. | System Description | 1 |  |
| 4. | List of Components | 2 |  |
| 5. | Memory Organization | 3 |  |
| 6. | I/O Organization | 3 |  |
| 7. | Software Flowchart | 5 |  |
| 8. | Appendix |  |  |
|  | a. ASM Code |  | 6 |
|  | b. Circuit Diagram |  | 23 |
|  | c. References |  | 24 |

PROBLEM STATEMENT

P23: System to be designed: An Intelligent Humidistat

System Description: A humidistat is supposed to be reset according to the outside temperature – as the outside temperature falls, the humidity level inside the house should be set lower. The purpose of this project is to develop a humidistat which senses the outside temperature and adjusts the humidity accordingly. Two sensors are required: outside temperature and inside humidity. Output is provided via a simple relay with the humidifier (presumably on the furnace) being on or off. Also, readings from the humidity and temperature sensors must be displayed on an LCD display. The entire system can be turned on or off using a single switch.

ASSUMPTIONS

The following are the assumptions made regarding the system:

- The outside temperature is between -50° C and 49° C.

- The humidifier turns on when the LED glows, and consequently, the humidifier turns off when the LED stops glowing.

- There is a linear relationship between temperature and humidity, i.e., for an increase in temperature by 1° C, there is an increase in relative humidity by 1% and at 0° C the humidity is 50%. For example – if the temperature is 17° C then the corresponding humidity should be 67% RH.

SYSTEM DESCRIPTION

The humidistat is supposed to change the humidity level inside a room according to the outside temperature. When the temperature goes low from the reference temperature (previously measured temperature), the humidifier is turned on through a solid-state relay. Then the humidity level is lowered by the humidifier and the humidity level is monitored until the humidity reaches a value corresponding to the current temperature. The humidifier is then turned off. And then the current temperature is made the reference value and the whole process is repeated again. The sensors are mounted outside the room and are open to the atmosphere. The humidity sensors are mounted inside the room. The humidity sensor measures the humidity in % Relative Humidity. The sensors give analog output. These outputs are converted to digital form through A/D converters. In the memory, there is a look up table which stores the % RH values obtained from the linear relationship for the corresponding temperatures in the temperature range.

LIST OF COMPONENTS USED

| Chip No. | Qty. | Chip | Purpose |
| --- | --- | --- | --- |
| 8086 | 1 | Microprocessor | Central Processing Unit |
| 6116 | 2 | SRAM | Used to store the temporary data (like |
|  |  |  | temperature values, stack, etc.) |
| 2732 | 2 | EPROM | Erasable Programmable Read Only |
|  |  |  | Memory; in which the code resides |
| 74138 | 2 | 3:8 Decoder | To Select PPI (8255), and for memory |
|  |  |  | interfacing |
| 8255 | 2 | Programmable | Provides I/O ports for the other devices |
|  |  | Peripheral Interface |  |
| ADC0808 | 1 | Analog to Digital | Converts the analog voltage to its digital |
|  |  | Converter | equivalent |
| 74LS245 | 2 | 8-bit bidirectional | Buffering Data bus |
|  |  | buffer |  |
| LM016L | 1 | 16x2 alphanumeric LCD | Displays the current temperature and humidity |
| 74LS373 | 3 | 8-bit octal latches | Latching the address bus |

OTHER HARDWARE USED

- Logic Gates – These are primarily used for building decoding logic for memory interfacing and I/O interfacing.

- Tri-state buffer – It is used to generate interrupt vector numbers.

- Solid-State Relay – It is used as a switch to power on high voltage devices.

- LED – It is used to signal the activity of the humidifier.

- Potentiometers – They are used to simulate input from sensors.

- Switches – They are used to power off/on the system and the LCD.

MEMORY ORGANIZATION

Memory is divided into odd and even banks for word and byte transfer.

ROM:

ROM1 EVEN: 00000h – 01FFEh

ROM1 ODD: 00001h - 01FFFh

RAM:

RAM1 EVEN: 02000h – 02FFEh

RAM1	ODD: 02001h -- 02FFFh

The data segment starts at the address of 00000h

Code Segment begins at 00000h.

MEMORY AND ADDRESS MAP

| CHIP | A19 A18 A17 A16 |  |  |  | A15 A14 A13 A12 |  |  |  | A11 A10 A9 A8 A7 A6 A5 A4 |  |  |  |  |  |  |  | A3 A2 A1A0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EPROM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2732 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| From |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 00000h | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| To |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 01FFFh | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 1 |  | 1 1 |  | 1 | 1 | 1 | 1 |
| SRAM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6116 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| From |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 02000H | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 0 |  | 0 0 |  | 0 | 0 | 0 | 0 |
| To |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 02FFFh | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 1 |  | 1 1 |  | 1 | 1 | 1 | 1 |

I/O INTERFACING

The input and output devices of the system are connected to the processor using 8255 Programmable Peripheral Interfacing controllers. Here we connect the A0 and A1 of the 8255 to the A1 and A2 of the 8086’s address bus respectively and also we connected the of 8255 to the output of decoder 74139 for selecting the chip. Addresses for the chips are as follow (All have been given even address space):

I/O Mapping

| Address of 8255-1 port-A | : 00h |
| --- | --- |
| Address of 8255-1 port-B | : 02h |
| Address of 8255-1 port-C | : 04h |
| Address of 8255-1 control register | : 06h |
| Address of 8255-2 port-A | : 08h |
| Address of 8255-2 port-B | : 0Ah |
| Address of 8255-2 port-C | : 0Ch |
| Address of 8255-2 control register | : 0Eh |

PORT ADDRESS MAPS

| CHIP | A19 A18 A17 A16 |  |  |  | A15 A14 A13 A12 |  |  |  | A11 A10 A9 A8 A7 A6 A5 A4 |  |  |  |  |  |  |  | A3 A2 A1A0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8255 - 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| From |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 00000h | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| To |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 00006h | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 0 |  | 0 0 |  | 0 | 1 | 1 | 0 |
| 8255 - 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| From |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 00008H | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 0 |  | 0 0 |  | 1 | 0 | 0 | 0 |
| To |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0000Eh | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 0 |  | 0 0 |  | 1 | 1 | 1 | 0 |

SOFTWARE FLOWCHART

APPENDIX

- ASSEMBLY CODE

#make_bin#

#LOAD_SEGMENT=FFFFh#

#LOAD_OFFSET=0000h#

#CS=0000h#

#IP=0000h#

#DS=0000h#

#ES=0000h#

#SS=0000h#

#SP=FFFEh#

#AX=0000h#

#BX=0000h#

#CX=0000h#

#DX=0000h#

#SI=0000h#

#DI=0000h#

#BP=0000h#

temp_ref     db  0,1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,17,18,19,20,21,21,23,24,25,26,27,28,29,30,31,32,33,34,35,36,37,38,39,40,41,42,43,44,45,46,47,48,49,50,51,52,53,54,55,56,57,58,59,60,61,62,63,64,65,66,67,68,69,70,71,72,73,74,75,76,77,78,79,80,81,82,83,84,85,86,87,87,89,90,91,92,93,94,95,96,97,98,99,100

ADC_out      db  00h,02h,05h,08h,0ah,0dh,0fh,12h,14h,17h,1ah,1ch,1fh,21h,24h,26h,29h,2bh,2eh,30h,33h,36h,38h,3bh,3dh,40h,42h,45h,47h,4ah,4dh,4fh,52h,54h,57h,59h,5ch,5eh,61h,63h,66h,69h,6bh,6eh,70h,73h,75h,78h,7ah,7dh,7fh,82h,85h,87h,8ah,8ch,8fh,91h,94h,96h,99h,9ch,9eh,0a1h,0a3h,0a6h,0a8h,0abh,0adh,0b0h,0b2h,0b5h,0b8h,0bah,0bdh,0bfh,0c2h,0c4h,0c7h,0c9h,0cch,0cfh,0d1h,0d4h,0d6h,0d9h,0dbh,0deh,0e0h,0e3h,0e5h,0e8h,0ebh,0edh,0f0h,0f2h,0f5h,0f7h,0fah,0fch,0ffh

temp_t      db  ?                            ;current temperature

temp_h      db  ?                            ;current humidity

cmp_t       db  ?                            ;voltage for current temperature

neg_flag    db  00h                          ;check sign of temperature

origin      db  50                           ;voltage for 0 degree temperature

jmp     strt

db     1024 dup(0)

strt:      cli

;intialize ds, es,ss to start of RAM

mov       ax,0000h

mov       ds,ax

mov       es,ax

mov       ss,ax

mov       sp,0FFFEH

; initializing 8255

sti

mov al,88h	; control word for 8255(for LCD)

out 06h,al

mov al,89h  ; control word for 8255(for ADC)

out 0Eh,al

mov al,00h 	;default low output for PC0

out 0ch,al

;initializing LCD

call dly_minor

mov al,04h

out 02h,al

call dly_minor

mov al,04h      ; to make rs=0 and r/w=0

out 02h,al

mov al,38h      ;function set

out 00h,al

mov al,04h

out 02h,al

call dly_minor

mov al,00h      ;to make rs=0 and r/w=0

out 02h,al

call dly_minor

mov al,0Ch      ; display on

out 00h,al

mov al,04h

out 02h,al

call dly_minor

mov al,00h

out 02h,al

mov al,06h      ; set entry mode

out 00h,al

call dly_minor

mov al,04h

out 02h,al

call dly_minor

mov al,00h

out 02h,al

mov al,4ch

out 00h,al

call dly_minor

start:	call    idle

call    clear_LCD

call    hello_world

call    dly_std

seq:  call    getHmd

call    getTemp

call    clear_LCD

call    dly_std

call    display_lcd

mov     al,cmp_t

mov     bl,temp_h

cmp     al,bl

ja      inc_hum

jb      dec_hum

call    idle

jmp     repeat

inc_hum:    call    inc_hmd

jmp     repeat

dec_hum:    call    dec_hmd

jmp     repeat

repeat:    call    dly_major

jmp     seq

dly_minor proc	near

mov    	cl,30

aa:

dec 	cl

jnz 	aa

ret

dly_minor endp

dly_major proc	near

mov	cx,0ffffh

bb:

dec 	cx

jnz 	bb

ret

dly_major endp

dly_std proc	near

mov	cx,5555h

st:

dec 	cx

jnz 	st

ret

dly_std endp

getTemp    PROC    NEAR                 ;get temperature through ADC

mov     al,00h

out     0eh,al    ; PC0=0

call    dly_major

mov     al,82h

out     0eh,al

in      AL,0AH

lea     si,ADC_out

lea     di,temp_ref

dec     si

cc:     inc     si

cmp     al,[si]

jnz     cc

sub     si,offset ADC_out

add     di,si

mov     al,[di]

mov     cmp_t,al

cmp     [di],50

jge     pos

mov     neg_flag,01h       ;for negative temperature

mov     al,[di]

mov     origin,50

sub     origin,al

mov     al,origin

mov     temp_t,al

jmp     con

pos:    mov     neg_flag,00h       ;for positive temperature

mov     al,[di]

sub     al,50

mov     temp_t,al

con:    call    CONVBCD

ret

getTemp    ENDP

getHmd     PROC    NEAR                    ;get humidity through ADC

mov     al,01h

out     0eh,al    ; PC0=1

call    dly_major

mov     al,82h

out     0eh,al

in      al,0aH

lea     si,ADC_out

lea     di,temp_ref

dec     si

dd:     inc     si

mov     bl,[si]

cmp     al,bl

jnz     dd

sub     si,offset ADC_out

add     di,si

mov     al,[di]

mov     temp_h,al

call    CONVBCD

mov     dx,bx

ret

getHmd     ENDP

;increase humidity

inc_hmd   proc    near

mov     al,0eh

out     0eh,al    ;reset decrease humidity signal

mov     al,0dh

out     0eh,al    ;set increase humidity signal

ret

inc_hmd   endp

;decrease humidity

dec_hmd   proc    near

mov     al,0ch

out     0eh,al    ;reset increase humidity signal

mov     al,0fh

out     0eh,al    ;set decrease humidity signal

ret

dec_hmd   endp

;idle humidifier when temperature and humidity are equal

idle       proc    near

mov     al,0eh

out     0eh,al

mov     al,0ch

out     0eh,al

ret

idle       endp

clear_LCD proc	near

mov al,00h

out 02h,al

call dly_minor

mov al,01h			;Clear LCD display

out 00h,al

call dly_minor

mov al,04h

out 02h,al

call dly_minor

mov al,00h

out 02h,al

RET

clear_LCD endp

hello_world proc	near

mov al,0A0h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints Space

mov al,0A0h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints Space

mov al,0A0h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints Space

mov al,48h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints H

mov al,65h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints e

mov al,6ch

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints l

mov al,6ch

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints l

mov al,6fh

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints o

mov al,0A0h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints Space

mov al,57h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints W

mov al,6fh

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints o

mov al,72h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints r

mov al,6ch

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints l

mov al,64h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints d

ret

hello_world endp

display_lcd   PROC    NEAR   ;Display temperature and humidity on LCD

mov al,54h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'T'

mov al,65h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'e'

mov al,6dh

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'm'

mov al,70h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'p'

mov al,65h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'e'

mov al,72h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'r'

mov al,61h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'a'

mov al,74h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 't'

mov al,75h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'u'

mov al,72h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'r'

mov al,65h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'e'

mov al,0A0h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints Space

cmp neg_flag,00h

jz  hh

mov al,2dh

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints '-'

jmp nn

hh:     mov al,0A0h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints Space

nn:    	mov al,bh

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints number stored in bh

mov al,bl

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints number store in bl

mov al,0DFh

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'DEG'

mov al,43h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'C'

mov al,0A0h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints Space

mov al,0c0h

out 00h,al

mov al,04h

out 02h,al

call dly_minor

mov al,00h

out 02h,al

call dly_minor

mov al,48h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'H'

mov al,75h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'u'

mov al,6dh

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'm'

mov al,69h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'i'

mov al,64h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'd'

mov al,69h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'i'

mov al,74h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 't'

mov al,79h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints 'y'

mov al,0A0h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints Space

mov al,0A0h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints Space

mov al,0A0h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints Space

mov al,0A0h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints Space

mov al,0A0h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints Space

mov al,dh

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints character in dh

mov al,dl

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints character in dl

mov al,25h

out 00h,al

call dly_minor

mov al,05h

out 02h,al

call dly_minor

mov al,01h

out 02h,al  ;prints '%'

ret

display_lcd    endp

CONVBCD PROC 	NEAR                    ;convert binary to bcd

mov     bh,0ffH

BACK1:  INC     BH

SUB     AL,0AH

JNC     BACK1

ADD     AL,0AH

MOV     BL,30H

ADD     BH,BL

ADD     BL,AL

RET

CONVBCD ENDP

- CIRCUIT DIAGRAM

- REFERENCES

A 16x2 LCD was used. The following were used to understand its working.

- https://panda-bg.com/datasheet/2134-091834-LCD-module-TC1602D-02WA0-16x2-STN.pdf

- https://www.csus.edu/indiv/p/pangj/class/lcd/instruct.html